===================================

Assign a Name to the Host Device

===================================

Configure passwords

Securing user EXEC mode access:

Securing privileged EXEC mode access:

Securing VTY line access:

PC0> telnet Sw-Floor-1-ip

===================================

Save Configurations

Configuration Files:

To save changes made to the running configuration to the startup configuration file, use: copy running-config startup-config

===================================

Switch Virtual Interface Configuration:

IP is assigned to switch as a whole object, not in a specific port.

VLAN is a virtual port. By default, all interfaces of a switch fall under the default VLAN 1.

===================================

Configure Initial Router Settings

Basic Router Configuration Example:

R1(config)# hostname R1

R1(config)# enable secret class

R1(config)# line console 0

R1(config-line)# password cisco

R1(config-line)# login

R1(config-line)# line vty 0 4

R1(config-line)# password cisco

R1(config-line)# login

R1(config-line)# exit

R1(config)# service password encryption

R1(config)# banner motd # message #
R1(config)# exit

R1# copy running-config startup-config

===================================

Configure Interfaces

Configure Router Interfaces:

Router(config)# interface type-a-number
Router(config-if)# description description-text
Router(config-if)# ip address ipv4-address subnet-mask
Router(config-if)# no shutdown

For Serial port [ONLY in the router with the ‘clock’ sign]:

int Se0/0

ip  address   100.1.1.1   255.255.252.0

clock rate 64000

no shutdown

Setting the default gateway

SW1(config)# ip default-gateway 172.16.1.1

Verify Interface Configuration:

R1#  show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0/0   192.168.10.1    YES manual up                    up

GigabitEthernet0/0/1   209.165.200.225 YES manual up                    up

Vlan1                  unassigned      YES unset  administratively down down

Configure Verification Commands:

show interfaces    Displays statistics for all interfaces on the device. Only displays the IPv4 addressing information.

show ip interfaces    Displays the IPv4 statistics for all interfaces on a router.

===================================

VLAN Config

Overview of VLANs:

VLAN Creation [in all switches/routers]:

Switch# conf t

Switch(config)# VLAN 10

Switch(config-vlan)# name CSE

Switch(config)# VLAN 20

Switch(config-vlan)# name ME

VLAN Port Assignment Commands [in all switches/routers]:

Switch# conf t

Switch(config)# int fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config)# int fa1/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Verify VLAN info:

Lists information about the VLANs:

SW1# show vlan {brief | id | name | summary}

Place interface back in VLAN 1:

Delete VLANs:

Delete VLANs with the no vlan vlan-id

Caution: Before deleting a VLAN, reassign all member ports to a different VLAN.

- Delete all VLANs with the delete flash:vlan.dat or delete vlan.dat commands.

- Reload the switch when deleting all VLANs.

===================================

VLAN Trunks

Trunk Configuration Commands [in all switches/routers]:

int g0/1

switchport mode trunk

switchport trunk native vlan 1

switchport trunk allowed vlan 10,20,1

Verify Trunk Configuration:

Switch# show int fa0/1 switchport

Lists all the trunk ports on a switch including the trunk allowed VLANs:

SW1# show interfaces trunk

===================================

Inter-VLAN Routing

Configure Legacy Inter-VLAN Routing::

Switch configuration:

Router Interface Configuration:

Configure Router-on-a-Stick::

Switch:

Router:

===================================

Routing

Static Default Routing [In BOTH NAT router and external router]:

ip route  0.0.0.0  0.0.0.0  Se0/0

[If this router doesn’t recognize any destination address, it directs the traffic to Se0/0 interface]

Static Routing:

ip route network-address-that-is-not-neighbor-of-this-router   subnet-mask  interface-to-reach-that-network-from-this-router

R1# show ip route static    Shows routes learned via static routing only

Dynamic Routing:

router rip

network network-address-that-is-neighbor-of-this-router

show ip route    Displays the contents of the IP routing tables stored in RAM.

R1# show ip route rip    Shows routes learned via RIP only

===================================

ACL [ACL rules per interface => inbound, outbound (From Router’s perspective); An ACL must have at least one permit statement otherwise all traffic will be denied due to the implicit deny ACE statement.]

Standard ACL [Closest to destination; Source IP, Wildcard-mask]::

Configure Numbered Standard IPv4 ACL [In config mode]:

ip access-list 10 permit 192.168.10.64 0.0.0.15 [192.168.10.64 to 192.168.10.79]

ip access-list 10 deny any (default)

ip access-list 10 permit host 192.168.10.10 [host means ‘0.0.0.0’; so this list permits ONLY the device with ip 192.168.10.10]

ip access-list 10 deny 192.168.10.0 0.0.0.255

[and denies all other ips]

int g0/0

ip access-group 10 in

Configure Named Standard IPv4 ACL[In config mode]:

ip access-list standard PERMIT-ACCESS

permit host 192.168.10.10

deny 192.168.10.0 0.0.0.255

int s0/0

ip access-group PERMIT-ACCESS out

Permit / Deny All IP:

R1(config-ext-nacl)# permit any [permit all except the ones denied with ‘deny’ command]

Extended ACL[Closest to source; Protocol, Src IP, Wildcard-mask, Dest IP, Wildcard-mask, Port]:

Configure Extended ACLs [Named]:

R1(config)# ip access-list extended FTP-FILTER

R1(config-ext-nacl)# permit tcp 192.168.10.0   0.0.0.255  any eq ftp

R1(config-ext-nacl)# permit tcp 192.168.10.0   0.0.0.255  any eq ftp-data

R1(config-ext-nacl)# permit tcp 192.168.10.0   0.0.0.255  any eq www

[permit any device from network 192.168.10.0 to connect to any destination IP with ports ftp,ftp-data,www]

R1(config-ext-nacl)# permit tcp 192.168.10.0   0.0.0.255  host 100.100.100.3 eq ftp

[permit any device from network 192.168.10.0 to connect to the device 100.100.100.3  with port ftp]

int s0/1

ip access-group FTP-FILTER out

Permit / Deny All IP:

R1(config-ext-nacl)# permit ip any any    [permit all except the ones denied with ‘deny’ command]

R1(config-ext-nacl)# deny ip any any [default]

Configure Extended ACLs [Numbered]:

Delete an ACL::

In Config Mode:

no ip access-list extended FTP-FILTER   Extended ACL

no ip access-list standard PERMIT-ACCESS    Standard ACL

no access-list 10	Both

In Interface Mode [Both extended and standard]:

int s0/0

no ip access-group FILTER out

Verify ACL:

R1# show access-lists

R1# show ip access-list   Shows all ACLs configured on a router with counters at the end of each statement

R1# show ip access-list 101   Shows only the specified ACL

R1# show ip interface g0/0   verify the ACL on the interface and the direction in which it was applied.

===================================

Static NAT

Configure Static NAT [In NAT router]:

R2(config)# ip nat inside source static 192.168.10.254 209.165.201.5

R2(config)#

R2(config)# interface serial 0/1/0

R2(config-if)# ip address 192.168.1.2 255.255.255.252

R2(config-if)# ip nat inside

R2(config-if)# exit

R2(config)# interface serial 0/1/1

R2(config-if)# ip address 209.165.200.1 255.255.255.252

R2(config-if)# ip nat outside

Verify Static NAT [In NAT router]:

R2# show ip nat translations

Pro  Inside global       Inside local       Outside local     Outside global

---  209.165.201.5       192.168.10.254     ---               ---

Total number of translations: 1

R2# show ip nat translations

R2# show ip nat statistics

Total active translations: 1 (1 static, 0 dynamic; 0 extended)

Outside interfaces:

Serial0/1/1

Inside interfaces:

Serial0/1/0

Hits: 4  Misses: 1

(output omitted)

===================================

Dynamic NAT

Configure Dynamic NAT [In NAT router]:

Clear Dynamic Translation Entry:

===================================

PAT

Configure PAT to Use a Single IPv4 Address [previous ACL 1 is used]:

R2(config)# ip nat inside source list 1 209.165.200.225 overload

R2(config)# access-list 1 permit 192.168.0.0 0.0.255.255

R2(config)# interface serial0/1/0

R2(config-if)# ip nat inside

R2(config-if)# exit

R2(config)# interface Serial0/1/1

R2(config-if)# ip nat outside

R2(config)# ip nat pool NAT-POOL2 209.165.200.226 209.165.200.240 netmask 255.255.255.224

R2(config)# access-list 1 permit 192.168.0.0 0.0.255.255

R2(config)# ip nat inside source list 1 pool NAT-POOL2 overload

R2(config)# interface serial0/1/0

R2(config-if)# ip nat inside

R2(config-if)# interface serial0/1/0

R2(config-if)# ip nat outside

Configure PAT to Use an Address Pool:

Verify PAT:

R2# show ip nat translations